c************************************************************

c    Directivity

c ************************************************************

c   This is a MATLAB and FORTRAN  based  program that computes the:

c

c	I.	Radiated power

c	II.	Maximum directivity

c

c   of any antenna.  The maximum directivity is calculated using the trailing

c   edge method in increments of 1o in theta and 1o in phi.

c

c	**Input parameters:

c	1. TL, TU:	Lower and upper limits in theta (in degrees)

c	2. PL, PU:	Lower and upper limits in phi (in degrees)

c	3. F(THETA, PHI): the radiation intensity function

c

c	**Note:

c	The radiation intensity function F must be provided for a given

c                antenna, and should be inserted into the subprogram U.

c

c	**Example:

c	If the antenna is radiating only in the upper hemisphere, the

c               limits on theta are 0 (degrees) and 90 (degrees) and the limits in

c	phi are 0 (degrees) and 360 (degrees)

c

c************************************************************