%****************************************************************

%   Synthesis

% ***************************************************************

%   This is a MATLAB based program that:

%

%   Implements the various synthesis methods presented in Chapter 7 of

%   Antenna Theory: Analysis and Design.

%

%  Specifically, the:

%

%    A. Schelkunoff Polynomial

%    B. Fourier Transform

%    C. Woodward-Lawson

%    D. Taylor (both Tschebyscheff-Error and One-Parameter)

%

%   methods are used to synthesize line sources and/or linear arrays

%   (where applicable).

%

%    Input Data

%   The user is guided in entering the correct input data through interactive

%   questions with built-in error checking.

%

%   Typical:

%

%    A. Schelkunoff Polynomial

%    I.  Option 1:  Compute polynomial for specific nulls

%			a. Specify spacing between elements (in wavelengths)

%			b. Specify number of desired nulls

%			c.  Specify the angles of each of the nulls (in degrees)

%    II.  Option 2:  Compute nulls for a specified polynomial

%			a.   Specify spacing between elements (in wavelengths)

%			b.  Specify progressive phase shift beta (in degrees)

%			c.  Enter the specified polynomial; choose of the following:

%				1.  Enter vector of polynomial coefficients in

%                                           descending form

%				---------------------------------------------------

%				Example: z^3-z^2+z-1 should be input as [1 -1 1 -1]

%				---------------------------------------------------

%				2. Specify polynomial as a function of z

%				--------------------

%				Example: z^3-z^2+z-1

%				--------------------

%				3. Specify roots of the desired polynomial

%				--------------------

%				Example(4 roots): +j, +1, -j, 0.707 + 0.707j

%				--------------------

%

%   B.  Fourier Transform

%    	I.  Option 1:  Line Source

%			a. Specify length of line source (in wavelengths)

%			b. Choose one of the following:

%				1. Read SF from file (angles must be in degrees and SF

%	                                   in linear scale)

%				    Note: SF function cannot be explicitly expressed as

%			              a function of theta. For example: A rectangular pulse.

%				2. Specify SF as a function

%				    Note: SF function can be explicitly expressed as a

%                                          function of theta.  Give SF as a function of x (x=theta).

%                                                          Use ^ for all powers and don't use quotes.

%					-------------------------------------

%					For example: sin(x)

%					--------------------------------------

%   	II.  Option 2:  Linear Array

%			a. Give number of elements for the array. Must be an

% 			    odd number since there is a non-zero dc value)

%			b. Choose one of the following:

%				1. Read AF from file (angles must be in degrees and AF

%				    in linear scale)

%				    Note: AF function cannot be explicitly expressed as

%				    a function of theta. For example: A rectangular pulse.

%				2. Specify AF as a function

%				    Note: AF function can be explicitly expressed as a

%				    function of theta.  Give AF as a function of x (x=theta).

%					Use ^ for all powers and don't use quotes.

%					-------------------------------------

%					For example: sin(x)

%					-------------------------------------

%    C.  Woodward-Lawson

%	I.  Option 1:  Line Source

%			a. Specify length of line source (in wavelengths)

%			b. Choose one of the following:

%				1. Read SF from file (angles must be in degrees and SF

%				    in linear scale)

%				    Note: SF function cannot be explicitly expressed as a

%                                          function of theta. For example: A rectangular pulse.)

%				2. Specify SF as a function

%			              Note: SF function can be explicitly expressed as a

%                                          function of theta.  Give SF as a function of x (x=theta).

%                                                Use ^ for all powers and don't use quotes.

%					-------------------------------------

%					For example: sin(x)

%					--------------------------------------

%				3.  Select even and/or odd samples

%					1. Even samples

%					2. Odd samples

%					3. Even & Odd samples

%   	II.  Option 2:  Linear Array

%			a. Give number of elements for the array

%			b.  Specify the spacing between the elements (in wavelengths)

%			c. Choose one of the following:

%				1. Read AF from file (angles must be in degrees and AF

%				     in linear scale)

%				     Note: AF function cannot be explicitly expressed as

%				     a function of theta. For example: A rectangular pulse.

%				2. Specify AF as a function

%				    Note: AF function can be explicitly expressed as a

%				    function of theta.  Give AF as a function of x (x=theta).

%					Use ^ for all powers and don't use quotes.

%					-------------------------------------

%					For example: sin(x)

%					-------------------------------------

%				3.  Select even and/or odd samples

%					1. Even samples

%					2. Odd samples

%					3. Even & Odd samples

%

%    D.  Taylor

%	I.  Option 1:  Taylor Tschebyscheff-Error

%			a.  Specify length of line source (in wavelengths)

%			b.  Specify desired constant side lobe level (in -dB)

%			c.  Specify number of side lobes of the same constant level (nbar)

% 	II.  Option 2:  Taylor One-Parameter

%			a.  Specify length of line source (in wavelengths)

%			b.  Specify desired constant side lobe level (in -dB)

%

%****************************************************************

%    Output Data:

%

%   A. Various plots of:

%	a.  Amplitude patterns (polar and rectangular forms)

%	b.  Source excitation distribution (amplitude and phase)

%   B.  Directivity

%   C.  Files of input and output data

%   D. Patterns and distributions are saved both in MATLAB's binary.mat format as

%        well as user-specified text files.

%*********************************************************************