*************************************************************

%   Gamma

% **********************************************************

%   This is a MATLAB program to analyze and design a Gamma-Match.

%

%   The Gamma-Match connection (which is a special case of the T-match,

%   and thus a particular case of the folded dipole) is employed as a

%   broadband matching technique to feed a dipole from a mismatched (and

%   possibly unbalanced) transmission line (such as a coaxial line).  The

%   inner gamma rod (usually with smaller radius) is connected to the

%   transmission line at one end and is tapped at one location to the outer

%   dipole (or antenna), the tap point set a distance l'/2 from its center.  The

%   gamma match and dipole are separated from each other by a small

%   distance, s.  A capacitor, whose purpose is to resonate the circuit and

%   balance the line, is placed at the feed point of the gamma rod.

%  (See Figure 9.22 from the book Antenna Theory by Balanis.)

%

%  This MATLAB program for a gamma-match connection is designed to

%  calculate the values of l'/2 and C, which are, respectively, the length of

%  the gamma rod (fed by the transmission line) and the capacitance of the

%  capacitor at the junction of the transmission line and the gamma rod.

%  The program assumes the geometry of the gamma rod-dipole is fixed

%  (other than the length of the gamma rod).  The user is prompted to select

%  from one of two options:

%

%   Either:  A dipole antenna whose length is known

%   Or:	An arbitrary antenna whose input impedance is known.

%

%   For either case the user is prompted to input the values of the two radii

%   (whether the gamma rod plus dipole or the antenna with gamma-match

%   connection) and the separation between the two elements.  For the two-dipole

%   case, the user inputs the length of the outer dipole; for the antenna-T-match

%   arrangement the user inputs the impedance of the antenna

%   (its real and imaginary parts separately).

%   In addition, the user inputs the characteristic impedance (which is

%   assumed to be real-valued) of the feed transmission line; this value is

%   denoted Zc.  The user must also input the frequency (in MHz) at

%   which the dipole is to resonate.

%

%-------------------------------WARNING!!----------------------------------------

%   This program will fail to yield any satisfactory answer if the

%  transmission line characteristic impedance, Zc, is too large.  In

%  particular, if Zc exceeds Zo/g2, the calculated output length l'/2 will

%  be a complex number; the variables Zo and g2 are calculated within

%  the program, and their values depend on the geometry specified by

%  the user (i.e., on the input values a, a', s, and l).  Furthermore, if Zc

%  approaches (from below) too closely the critical value of Zo/g2, the

%  output value of l'/2 may be real, but it will be negative.

%

%  The center-to-center separation, s, of the gamma rod and the outer

%  dipole must be sufficiently large to satisfy the constraint

%  s^2 > (a^2 + (a')^2).  This constraint should never pose a problem

%  for realistic geometries, but must be heeded to prevent the user from

%  entering too small a value of s.

%-----------------------------------------------------------------------------------------

%

%  The program computes and outputs the following values:

%

%  Effective radius of the gamma rod-dipole configuration, ae

%  Dipole (antenna) input impedance, Za

%  Characteristic impedance of the element pair, Zo

%  Transmission line mode impedance, Zt

%  Total input resistance, Rin

%  Total input reactance, Xin

%  Length of the gamma rod, l'/2 = lgamma

%  Capacitance of the resonating capacitor, C

%

%  The program also outputs the values of the parameters input by the user:

%  Outer dipole (antenna) radius, a

%  Gamma-match wire radius, a'

%  Center-to-center wire separation, s

%  Outer dipole length, l  **or**  the impedance of the antenna

%  Characteristic impedance of the feed transmission line, Zc

%  Frequency at which to resonate, f

%

%************************************************************