**********************************************************************

%   Quarterwave

%  ***************************************************************

%	This program is a MATLAB  based software that makes use of the binomial

%	and tschebyscheff design method of the multiple-section quarter-wavelength

%	trasformer for impedance matching.

%

%	The general idea of the method is to insert N quarter-wavelength subsection

%	of transmission lines between the impedance load and the characteristic

%       impedance of the input transmission line to provide an impedance frequency

%       response with Binomial/maximally-flat and Tschebyscheff/equal-ripple

%       characteristics.

%

%	To utilize the method, the user must do the following:

%	1.  Select which method to use:

%		a.  Binomial

%		b.  Tschebyscheff

%	2.  Specify the:

%		a.  Characteristic impedance value of the input transmission line

%		b.  Load impedance. Indicate whether the impedance is real or complex

%		c.  Number of the quarter-wavelength subsections

%		d.  Fractional bandwidth or

%		d''. Maximum tolerable reflection coefficient (specify d or d''; not both)

%

%	The program computes the:

%	1.  Intrinsic reflection coefficients at each interface

%	2.  Characteristic impedances of each quarter-wavelength

%		subsection transmission line

%	3.  Maximum tolerable reflection coefficient or

%	3''. Fractional bandwidth (computes 3 or 3''; not both)

%	4.  Maximum input VSWR based on the maximum tolerable

%            reflection coefficient

%

%	The program plots the magnitude of the input reflection coefficient

%	as a function of the relative frequency 0 <= f/f0 <= 2.

%

%***************************************************************