*************************************************************

%    Tee

%************************************************************

%   The T-Match connection (which is a particular case of the folded dipole) is

%   employed as a broadband matching technique to feed a dipole from a

%   mismatched, but most likely unbalanced, transmission line (like a twin-lead

%   line).  The inner, shorter length (and usually smaller radius) dipole is connected

%   to the transmission line at its center and is tapped at two locations to the longer

%   (outer) dipole, each tap point a distance l'/2

%   from its center.  The two dipoles are separated from each other by a small

%   distance, s.  Two capacitors (of equal capacitance), whose purpose is to resonate

%   the circuit and balance the line, are placed at the feed points of the inner dipole.

%   (See Figure 9.21 from the book Antenna Theory by Balanis)

%

%   This MATLAB program for a T-Match connection is designed to calculate the

%   values of L' and C, which are, respectively, the length of the inner dipole (fed by

%   the transmission line) and the capacitance of each of the two capacitors at the

%   junction of the transmission line and the inner dipole.

%

%   The program assumes the geometry of the dipoles is fixed (other than the length

%   of the inner dipole).  The user is prompted to select from one of two options:

%

%   Either:   A dipole antenna whose length is known

%   or:         An arbitrary antenna whose input impedance is known.

%

%   For either case the user is prompted to input the values of the two radii (whether

%   the two dipoles or the antenna with T-match connection) and the separation

%   between the two elements.  For the two-dipole case, the user inputs the length of

%   the outer dipole; for the antenna-T-match arrangement, the user inputs the

%   impedance of the antenna (its real and imaginary parts separately).

%

%   In addition, the user inputs the characteristic impedance (which is assumed to be

%   real-valued) of the feed transmission line; this value is denoted Zc.  The user

%   must also input the frequency (in megahertz) at which the dipole is to resonate.

%

%

% -------------------------------WARNING!!-----------------------------------------------

%   This program will fail to yield any satisfactory answer if the transmission lin

%   characteristic impedance, Zc, is too large.  In particular, if Zc exceeds Zo/g2, the

%   calculated output length l' will be a complex number; the variables Zo and g2 are

%   calculated within the program, and their values depend on the geometry

%   specified by the user (i.e., on the input values a, a', s, and l).

%   Furthermore, if Zc approaches (from below) too closely the critical value of

%   Zo/g2, the output value of l' may be real, but it will be negative.  The center-to-

%   center separation, s, of the inner and outer dipoles must be sufficiently large to

%   satisfy the constraint s^2 > (a^2 + (a')^2).  This constraint should never pose a

%   problem for realistic geometries, but must be heeded to prevent the user from

%   entering too small a value of s.

%-----------------------------------------------------------------------------------------

%

%   The program computes and outputs the following values:

%	1.  Effective radius of the dipole-dipole configuration, ae

%	2.  Dipole input impedance, Za

%	3.  Characteristic impedance of the dipole pair, Zo

%	4. Transmission line mode impedance, Zt

%	5.  Total input resistance, Rin

%	6.  Total input reactance, Xin

%	7.  Length of the inner dipole, l'

%	8.  Capacitance of each resonating capacitor, C

%

%   The program also outputs (returns) the values of the parameters input

%   by the user:

%	1.  Outer dipole radius, a

%	2.  T-match wire radius, a'

%	3.  Center-to-center wire separation, s

%	4.  Outer dipole length, l  **or** the impedance of the antenna

%	5.  Characteristic impedance of the feed transmission line, Zc

%	6.  Frequency at which to resonate, f

%

********************************************************************