c************************************************************

c   Yagi_Uda

c ************************************************************

c   This is a MATLAB and FORTRAN  program that computes, for the

c   Yagi-Uda antenna array, the:

c

c	I.	Far-zone E- and E-plane amplitude patterns (in dB)

c	II.	Directivity of the array (in dB)

c	III.	E-plane half-power beamwidth (in degrees)

c	IV.	H-plane half-power beamwidth (in degrees)

c	V.	E-plane front-to-back ratio (in dB)

c	VI.	H-plane front-to-back ratio (in dB)

c

c    The program plots the:

c     I.    Normalized current at the center of each element

c     II.   Normalized E- and H-plane amplitude polar patterns (in dB)

c

c    The program is based on Pocklington's Integral Equation formulation of

c    Section 10.3.3, Equations (10-42)-(10-65a).  M entire domain cosinusoidal

c    (Fourier) basis modes are used for each of the antenna elements.

c

c	**Input parameters:

c	1.	M	=	number of entire domain basis modes

c	2.	N	=	number of antenna elements

c	3.	l	=	length of each element ln  (in )

c	4.	Alpha	=	radius of each element a  (in )

c	5.	s	=	separation between the elements sn  (in )

c

c	**Notes:

c	1.	Refer to Figure 10.19 for the geometry.

c	2.	Driven element is at the origin.

c	3.	First element (N = 1) is the first director.

c	4.	Reflector is the N-1 element; only one reflector.

c	5.	Driven element is N.

c	6.	The radius of all the elements is the same.

c

c*************************************************************