INSTRUCTIONS FOR USING THE LOG-PERIODIC

DIPOLE ARRAY DESIGN PROGRAM

Note:  This document should be viewed from an editor which does

not use proportional fonts such as the DOS EDIT editor or the

Windows NOTEPAD editor.  Alternatively, it can be printed or

viewed with a non-proportional font such as Courier.  In this

manner, the various text figures can be fully appreciated.

This documentation file contains information for the correct

use of the log-periodic dipole array (LPDA) design program

included with the book, ANTENNA THEORY:  ANALYSIS AND DESIGN, by

Constantine A. Balanis.  When the design program is first run,

the many input variables can be confusing, especially since many

of them represent impedances - characteristic, load, source, and

desired input impedances.  Furthermore, the program utilizes

some approximations which need to be fully understood by the

user.  Only with this kind of detailed knowledge can the user

fully appreciate the difficulty of accurately modeling antennas.

Finally, many of the sources for the equations come directly

from this book, but others come from elsewhere.  For instance,

the transmission line theory from introductory electromagnetics

forms a key part of the analysis.  Therefore, the references for

this work are summarized at the end of the document.

This file contains ten parts as follows:

1.     Geometry definitions

1.1.  Antenna Geometry

1.2.  Coordinate Geometry

2.     Discussion of input parameters

2.1.  Design Parameters

2.2.  Analysis Parameters

3.     Algorithm development

3.1.  Self and Mutual Impedances

3.2.  Transmission Line Admittance Matrix

3.3.  Combining the Matrices

3.4.  Finding the Input and Termination Currents

3.5.  Finding the Critical Parameters

4.     Subtleties and assumptions

5.     Output parameters

6.     Verification and validation summary

7.     FORTRAN Compilation

8.     Credits

9.     References

1.  GEOMETRY DEFINITIONS

1.1.  ANTENNA GEOMETRY

The geometry utilized for the analysis largely corresponds

to the geometry of Figure 11.9(a) in the book.  FIGURE 1 shows

this geometry redrawn.

Element Number:

1  2   3   ...  N

|

|        |

|    |        |

|  |    |        |

x

->| R1|<-|    |        |

|<-R2->|    |        |

|<--- R3 -->|        |

|<------  RN  ------>|

|

Apex

FIGURE 1.  Array Geometry

Several important additions are not shown in this figure.

First, something must energize the antenna.  This source is

generally a voltage source with an internal resistance, Rs.

This voltage source is connected to the shortest element,

element 1, by means of a source transmission line which is

different from the transmission line (often referred to as the

"boom") which connects the antenna elements.  Often this source

transmission line is a coaxial cable.  The center conductor is

connected to one side of element 1, and the shield is connected

to the other side of element 1.  There are many subtleties

associated with this connection, but this analysis ignores them.

The effect of including the source resistance, Rs, is a

reduction in the antenna efficiency from 100% to something less.

For instance, let us assume the antenna has an input impedance

of 50 Ohms as measured at the input terminals of element 1.

Further assume the source transmission line characteristic

impedance is 50 Ohms so that it is matched to the antenna.  If

there were no source resistance, the antenna efficiency would be

100%.  Now assume that a 5 Ohm source resistance (internal to the

voltage generator) were present.  The resulting circuit can be

analyzed as shown in FIGURE 2.

|--- 5 Ohms --------------|

|   (source               |

|    resistance)          |

1V  O                       50 Ohms (antenna)

ac  |                         |

|                         |

|-------------------------|

FIGURE 2.  Equivalent Circuit Considering the

Antenna as a Load

The proportion of voltage transferred to the load (50 Ohms) is

50

%V = ------ 100% = 91%

50 + 5

The fraction of power received by the load is equal to (%V)**2 or

%P = (0.91)(0.91) 100% = 83%

83% is the antenna efficiency neglecting other sources of

inefficiency.  This efficiency factor results in a decrease in

gain from the published values, such as in Figure 11.13.  The moral

of this calculation is that design equations tend to yield the

best possible answer and that other mechanisms can degrade the

real-world results.  Only careful modeling and attention to

detail can prevent substandard performance.

The other side of the antenna, that is, the side with the

longest element, also has an additional feature:  a termination

impedance is added across the terminals of element N (the

longest element).  To understand why this is necessary, consider

first an antenna without a termination impedance.  Instead, the

transmission line is left open.  What happens if some energy,

injected at element 1, manages to continue down the antenna

transmission line past the active region to strike the open

circuit.  Transmission line theory says it reflects from the

open and travels back toward the source.  While this means more

energy could be radiated (the reflected energy has a second

chance to radiate), it also means that interference effects will

occur.  Experimentation with this program will show that this

interference will result in a design whose VSWR versus frequency

contains many spikes at particular frequencies.  In contrast, if

a termination impedance is added which has the same value as the

effective antenna transmission line characteristic impedance,

all energy which travels past the active region will be absorbed

by the load.  The resulting VSWR is much smoother than the

previous case.

Why would a designer choose one reflection elimination

technique over another?  Using a matched load is the best

solution in terms of performance, but often it is difficult,

undesirable, or not cost effective to solder a resistor across

the longest element's terminals.  The quarter-wave transformer

is cheaper and easier to construct and provides an improvement

over not doing anything.  However, it also makes the physical

size of the antenna longer, a possible disadvantage.

FIGURE 3 shows the resulting antenna geometry.

|

|        |

<- LLin ->   |    |        |<- LLout ->

__________|  |    |        |___________

Rs__________                  ___________Zout

->| R1|<-|    |        |

|<-R2->|    |        |

|<--- R3 -->|        |

|<------  R4  ------>|

|

Apex

FIGURE 3.  Geometry Showing the Source and

Termination Transmission Lines

and Impedances

In addition to the placements of the elements, sources, and

terminations, one must also consider the construction of the

antenna transmission line.  Because this transmission line

generally, though not always, provides a structure upon which to

mount the antenna elements, it is also called a "boom."  The

boom often consists of a twin lead transmission line made from

two copper tubes as shown in detail in Figure 11.9(d) in the

book.  This construction actually represents a departure from a

truly log-periodic design.  The geometry of truly frequency

independent antennas is a function of (apex) angle only[1].  For

instance, the truncation of the antenna at elements 1 and N is

not a function of the apex angle, Alpha, but rather of the

distances R1 and RN (see Figure 11.9(a) in the book).  The

result is an antenna which operates over a frequency band

(albeit a large one), not over all frequencies.  Therefore, the

spacing and diameters of the two tubes which form the boom

should increase linearly with distance from the apex.  At the

apex they should be spaced by zero and have zero diameter.  It

is also for this reason that the diameters of the elements

increase with distance from the apex.

Although maintaining constant element diameters can have a

noticeable effect on the pattern, maintaining constant boom

spacing and boom tube diameters has a very minor effect on the

pattern.  For this reason and for convenience, the boom

center-to-center spacing is fixed at SB, and the boom tube

diameters is fixed at DB.  These two parameters are sufficient

to calculate the characteristic impedance of the boom

transmission line (without the elements attached)[2][3].

1.2.  COORDINATE GEOMETRY

While the antenna geometry given in Section 1.1 is enough to

design the antenna, a coordinate system is needed for the

analysis.  FIGURE 4 shows the coordinate system for analysis.

Z axis

^

|

________ | ________

|

|

______ | ______

|

___ | ___

|

_ | _

|

Apex ----->O-------------->  y axis

X axis

(out of page)

FIGURE 4.  Coordinate System

Furthermore, the angle Phi is measured from the x axis toward

the y axis in the x-y plane.  The angle Theta is measured from

the z axis toward the x-y plane.

The E-plane is defined as the plane which contains the

electric field vectors and also the major axis (here, the z

axis) of the antenna.  Since the E-field develops where there is

a voltage drop (V = Integral(-E * dl) and since there is a

voltage drop from one side of each element to the other, the

E-plane is the y-z plane.  The H-plane also contains the z axis

and is perpendicular to the E-plane.  Therefore, the H-plane is

the x-z plane.  In terms of Phi and Theta, the E-plane has Phi

fixed at 90 or 270 degrees, and Theta is allowed to vary from

0 to 180 degrees on both sides of the z axis.  The H-plane has

Phi at 0 or 180 degrees and Theta is allowed to vary from 0 to

180 degrees on both sides of the z axis.  One important

consequence of these definitions is that the boresight of the

antenna is at Theta equal to 180 degrees, not Theta equal to

0 degrees.  (Here, "boresight" refers to the direction of the

mechanical axis of the antenna.  In other contexts, "boresight"

is the direction where the pattern is maximum.)

2.  DISCUSSION OF INPUT PARAMETERS

Now that we have defined the geometry, it is important to

precisely understand each term and its representation in the

program.  Some parameters can be calculated from others.

2.1.  DESIGN PARAMETERS

INPUT VARIABLES *

TITLE  = Design title

D0     = Desired gain

Fhigh  = Upper design frequency

Flow   = Lower design frequency

Rs     = Source impedance internal to the voltage generator

ZCin   = Characteristic impedance of the input transmission

line

Rin    = DESIRED input impedance, measured at the terminals

of element 1 (the shortest element)

LLin   = Line length of the input transmission line

Zout   = Termination impedance

LLout  = Line length of termination transmission line

LD     = Length to diameter ratio of antenna elements

Navail = Number of available wires or tubes for the elements

Davail = Diameters of available tubes for the elements

SB     = Spacing of boom tubes or wires

DB     = Diameter of boom tubes or wires

Tau    = Geometric ratio (see Section 11.4.2)

Sigma  = Spacing factor (see Section 11.4.2)

DESIGN VARIABLES **

L(n)   = Total length of element n

D(n)   = Diameter of element n

ZL(n)  = Location along the z axis of element n

ZO     = Characteristic impedance of antenna transmission

line

ZinA   = ACTUAL input impedance, measured at the terminals

of element 1 (the shortest element)

*  Note that some of the input variables listed in the book

are listed in Section 2.2., ANALYSIS PARAMETERS.

** Note that this list is somewhat abbreviated and that some

variables, such as Tau and Sigma, can be considered as

belonging to more than one category.

Now that each design parameter is defined, how does one go

about choosing values?  For starters, some parameters are

available for more precise modeling of a particular application

or design.  For this reason, the source impedance (Rs), input

line length (LLin), and the output line length (LLout) all can

be set to zero.  Furthermore, setting the option to not quantize

the element diameters forces the program to ignore Navail and

Davail.

Certain parameters must be known by the designer before the

design.  These include the characteristic impedance of the

source transmission line (ZCin), the frequency range (Fhigh and

Flow), and the desired gain (D0).  If the characteristic

impedance of the source transmission line (ZCin) is not known,

guess:  for most coaxial cable in the UHF band is 50 or 75 Ohms.

Since we want to match the antenna to this cable, the desired

input impedance (Rin) should be equal to the source transmission

line characteristic impedance (ZCin).  Selecting the desired gain

sets Tau and Sigma for an optimum design.  Alternatively,

selecting Tau and Sigma allows independent control for special

applications.  All other parameters will be calculated by the

program.

Since the program only estimates the transmission line

characteristic impedance (ZO) for the antenna to achieve the

desired input impedance (Rin), the actual input impedance (ZinA)

may not be correct.  For instance, assume that the actual input

impedance (ZinA) comes out to be 60 to 65 Ohms for a 50 Ohm

desired input impedance (Rin).  In this case, lower the desired

input impedance (Rin) so that the actual input impedance (ZinA)

comes out nearly correct (that is, approximately equal to ZCin).

More accurate estimates for ZO exist.[4][5]

Now that the design is complete and satisfactory, quantize

the element diameters to the available wire or tube diameters.

This quantization rounds each calculated element diameter to the

nearest available size.  The variable, Navail, tells how many

sizes are available.  Davail contains the available diameters.

Next, perturb the design by adding a source impedance of about

5 Ohms.  This should decrease the gain by about 1 dB for an

antenna with an input impedance (ZinA) 50 Ohms matched to a

50 Ohm source transmission line.

Finally, add a matched load (Zout) to suppress reflections

from the open-circuit termination.  Notice the resulting

decrease in VSWR at many frequencies.

2.2.  ANALYSIS PARAMETERS

In addition to the design parameters, the input screens

also ask for certain analysis parameters.  The user can select

single frequency E- and H-plane analyses, single frequency

custom plane analysis, and/or swept frequency analysis.  The

input parameters are as follows.

AFSEH  = Frequency for single frequency analysis of E- and

H- planes

AFSC   = Frequency for single frequency analysis of custom

plane

AFhigh = Upper analysis frequency for swept frequency

analysis

AFlow  = Lower analysis frequency for swept frequency

analysis

Phi    = Angle of custom plane (90 degrees equals E-plane,

0 degrees equals H-plane)

AFpowr = Number of frequency steps per octave

All parameters are self-explanatory except the last.  For

swept frequency analysis, the program starts at the lowest

analysis frequency, AFlow, and after each iteration increases

the analysis frequency by a certain amount until it exceeds the

upper analysis frequency.  The step size, AFstep, is defined by

the following relation.

AFstep = FMHz * 10 ** (1/AFpowr)

where FMHz is the current analysis frequency.  This relation has

the advantage of providing equally spaced points when frequency

is plotted on a logarithmic scale.  Increasing AFpowr increases

the resolution.

3.  ALGORITHM DEVELOPMENT

Some of the algorithms have already been explained, but for

the sake of clarity, all will be explained here.  There is

nothing fantastically difficult about any particular step of the

algorithm, but taken together there are so many topics that it

is easy to become confused.

Since the design of the antenna is covered in the book, it

will be omitted here.  Let us begin the discussion knowing the

topology of the design, as if we had an antenna in hand, so that

we are ready to analyze it.

There are several steps to the analysis of the antenna.

First, we must calculate the self and mutual impedances of each

antenna element.  These calculations tell us how each excited

element interacts with each other element and how well each

antenna radiates.  Next, we need to calculate the

characteristics of the antenna transmission line.  This

calculation accounts for the termination impedance (Zout) as

well as the source impedance (Rs).  It tells us how energy

propagates down the transmission lines.  Third, we need to solve

for the currents on each antenna element as well as the input

current (Iin) and voltage (Vin) and the termination current

(Iout) and voltage (Vout).  Finally, we calculate the gain,

VSWR, and other important parameters.

3.1.  SELF AND MUTUAL IMPEDANCES

The self and mutual impedances of radiating elements is a

subject of much research.  To understand the basic idea[8],

consider several radiating elements located in space.  Now

remove all elements except one, for instance element 1, and

excite it with a current.  The input impedance can be measured

as the ratio of the input voltage over the input current.  This

is the self impedance denoted Z11.  Now replace one of the other

elements, say element 2, and short its terminals together.

Recalculate the input impedance of element 1.  This is the

mutual impedance denoted Z12 (impedance of element 1 due to

element 2).  Repeat the process for every pair of antennas.

(The measurements can be roughly halved, because reciprocity

assures us that Z12 = Z21.  This means the resulting matrix is

symmetric.)

To obtain very accurate results, finite difference methods

and projection methods, which include moment methods and

finite element methods often are used.  Finite difference

methods approximate the governing differential equations by

mathematically dividing the surface into very small segments, then

approximating a derivative by the equation

f(x2) - f(x1)

df = ---------------

x2 - x1

For projection methods, a set of basis functions which satisfy

the boundary conditions are weighted and summed to form an

approximate answer.  For moment methods, the basis functions are

valid for the entire antenna surface.  For finite element

methods, the basis functions are valid over a small part of the

antenna surface.

An alternative approach, and the one used here, assumes

that the current distribution along each antenna varies

sinusoidally.  This assumption is valid for infinitesimally thin

dipoles sitting alone in space.  While this assumption is not

totally valid, the approximation is still a good one.

With this assumption, the self and mutual impedances can be

written as integral equations.  These equations can be

recast in terms of the sine and cosine integrals[9][10].  With

these equations, the symmetric matrix, called ZA in the program,

can be found.

3.2.  TRANSMISSION LINE ADMITTANCE MATRIX

Next, we must compute the characteristics of the antenna

transmission line.  Good documentation can be found in the

literature [6][7][8], although these references can

sometimes be hard to find.  Therefore, a summary will be

presented here.  We wish to create the same kind of data for the

transmission line as we did for the self and mutual impedances.

That is, we will excite the transmission line with a unit

voltage everywhere an antenna element attaches, one place at a

time.  The remaining places will be shorted.  This procedure is

derived directly from N-port admittance matrix theory.  The task

is to find the resulting current at each port (place of antenna

attachment).

FIGURE 5 illustrates the concept for an N element antenna

excited at port 4.  Only a few of the ports are shown.  The

distance between ports is given by the variable DZL.

PORT:  2        3        4        5

----------------------------------------------

|        |        +        |

|        |                 |

|        |                 |

|        |                 |

o o o   |        |        V        |    o o o

|        |                 |

|        |                 |

|        |                 |

|        |        -        |

----------------------------------------------

|<-DZL2->|<-DZL3->|<-DZL4->|

FIGURE 5.  Transmission Line Equivalent Circuit

For the following discussion, please refer to any standard

electromagnetics text.  For example, see [2][3].  Also see

[4][7][8].

It is obvious by inspection that given an excitation at

element 4, a current will flow only through the shorts at ports

3, 4, and 5.  Current will not flow at the other ports.  The

current at port 4 is the current through the source, V.  The

transmission line equation tells us that the impedance seen by

port 4 in the direction of port 3 is equal to

Z43 = j ZO tan(Beta * DZL3)

where ZO is the characteristic impedance of the line, and Beta

is the wavenumber.  The admittance (Y) is one over the impedance

so,

Y43 = -j YO cot(Beta * DZL3)

where YO = 1/ZO.  Similarly, the admittance seen by port 4 in

the direction of port 5 is

Y45 = -j YO cot(Beta * DZL4)

Therefore, the current flowing through port 4 (excited by a unit

voltage) is

Y43 + Y45 = I4 / 1

To find the current in port 3, the following general

transmission line equation is used.

V(DZL3) = V3 * cos(Beta*ZL3) + j * I3'' * ZO * sin(Beta*ZL3)

where V3 and I3'' are the voltage and current at port 3,

respectively.  (Note:  the primes do not represent

differentiation.)  Since port 3 is shorted, V3 = 0.

Furthermore, V(DZL3) is just the excitation voltage,

V4 = 1 Volt.  Therefore,

1 = j * I3'' * ZO * sin(Beta*ZL3)

or

I3'' / 1 = -j YO * csc(Beta*L)

However, this definition of I3'' is reversed from that of the

standard N-port definitions;  it flows out of the port instead

of in to it.  Therefore, we remove one prime ('') and change the

sign.

I3' / 1 = j YO * csc(Beta*L)

One step remains.  Since we arranged for a phase reversal by

crossing the wires in the transmission line [], we must account

for this by changing the sign again.  The final form for the

current is then

I3 / 1 = -j YO * csc(Beta*L)

The admittance seen in port 3 due to an excitation at port 4 is

then

Y34 = -j YO * csc(Beta*L)

Notice that the resulting matrix will be tri-diagonal.

That is, it will have non-zero elements only along its major

diagonal and the diagonals on either side of it.

The termination impedance is accounted when the admittance

seen by element N is calculated.  The transmission line equation

tells the equivalent impedance seen by element N in the direction

of the load.

The program does not use the admittance matrix to account for

the source transmission line and source resistance.  Instead,

it accounts for the effect of these items later after the solution

for the antenna is obtained.

The resulting tri-diagonal admittance matrix is called YT in

the program.

3.3.  COMBINING THE MATRICES

At this point we have one matrix, ZA, which describes how

the elements interact with each other.  Another matrix, YT,

describes how the transmission line propagates energy.  We wish

to combine the two.  Following [7], the connection of the

N-port antenna elements network (ZA) and the N-port transmission

line network (YT) amounts to connecting the two N-ports in

parallel.  Therefore we can write that

Iel = YA Vel

IT = YT VT

where Iel is the current at the input to the antenna elements

(A column vector)

YA is the inverse of ZA

(A square matrix)

Vel is the voltage at the input to the antenna elements

(A column vector)

IT is the current at the ports of the transmission line

(A column vector)

VT is the voltage at the ports of the transmission line

(A column vector)

Since the connection occurs in parallel, the total current is

then

I = Iel + IT

Here, I represents the excitation current.  For a log-periodic

dipole array, we excite the shortest element, element 1, only.

Therefore, I = [1 0 0 ... 0]T (T denotes transpose).  When

we make the connection, Vel = VT.  Therefore,

I = (YA + YT) Vel

(Here this text departs from [7].)  We want the voltages on each

antenna element measured at the port.  Therefore, we rewrite

this equation as

(ZAZT) = inverse of (YA + YT)

Vel = (ZAZT) I

From this,

Iel = YA Vel

The references [4][6][7] recast the equations slightly to make

them faster to evaluate.  For this program, the straightforward

approach is fast enough.

3.4.  FINDING THE INPUT AND TERMINATION CURRENTS

Now at last we have the antenna currents and voltages.

Before we calculate the critical parameters, let us find the

source and termination currents and voltages.  For this theory,

we again consult [2][3], and write

I(z') = IL * cos(Beta*z') + j * VL / ZO * sin(Beta*z')

V(z') = VL cos(Beta*z') + j * IL * ZO * sin(Beta*z')

Here, z' is a distance measured from the load to the source

VL is the voltage at the load

IL is the current at the load

ZO is the characteristic impedance of the transmission line

To find the source current and voltage, we reduce the antenna to

the source generator, Vin, and an equivalent impedance which

replaces everything but the source and the input transmission

line (ZinA).  The result is a diagram similar to FIGURE 2 where

the antenna becomes the load.  Therefore, we set

z'    = LLin (Length of source transmission line)

V(z') = Vin'

I(z') = Iin

VL    = Vel1

IL    = 1

ZO    = ZCin

Note that

ZinA = Vel1 / 1 Amp = Vel1

This is the input impedance measured at the shortest element of

the dipole array.  The result of the above substitutions is

Iin  = cos(Beta*LLin) + j * Vel1 / ZO * sin(Beta*LLin)

Vin' = Vel1 * cos(Beta*LLin) + j * ZO * sin(Beta*LLin)

These are explicit expressions for Iin and Vin'.  Vin' is the

voltage, not at the source, but after the current drop across

the source resistance is accounted as shown in Figure 6.

Iin->

|--- Rs -------------| + Vin'

|   (source          |

|  resistance)    (ZinAS - antenna impedance

Vin  O                  as seen by the

|                  voltage source)

|                    |

|--------------------| -

FIGURE 6. Vin, Iin, and Vin'

Rs + ZinAS

Therefore, Vin = ---------- Vin'

ZinAS

ZinAS can be determined from the transmission line equation

or by

ZinAS = Vin'/Iin

It is the antenna input impedance as seen by the source.

The power accepted by the antenna is

1

Pin = - (Iin) (Iin*) [real(ZinAS) + Rs]

2

where * indicates complex conjugate.

Note that confusion might exist about the definition of

the input impedance.  There are two definitions:  ZinA is the

input impedance measured at the shortest element of the

antenna array, and ZinAS is the input impedance as seen by the

voltage source.  Note that the magnitude of ZinAS will vary

with the length of the input transmission line.  The reason

for this is that for an antenna which is not precisely matched

to the characteristic impedance of the input transmission line,

a standing wave exists on this line.  For this reason, a design

engineer might be interested in both measures od input impedance.

Finding the termination current and voltage is similar.

This time the antenna is the source and the termination is the

load.  The above equations must be turned around to yield

Vout = VelN * cos(Beta*LLout) - j*IN'  * ZO*sin(Beta*LLout)

Iout = IN'  * cos(Beta*LLout) - j*VelN / ZO*sin(Beta*LLout)

where

VelN is the voltage present at the terminals of the last

(longest) element

LLout is the length of the termination transmission line

ZO is the termination transmission line characteristic

impedance (equal to the antenna transmission line

characteristic impedance)

and where

IN' = VelN / ZR

Zout + j * ZO   * tan(Beta * LLout)

ZR = ZO * -------------------------------------

ZO   + j * Zout * tan(Beta * LLout)

Zout is the termination impedance.

3.5.  FINDING THE CRITICAL PARAMETERS

From the currents and voltages just calculated, we can find

all the critical parameters of the design.  Before we do, it is

desireable to take care of one small point.  It is convenient to

normalize all the currents and voltages to 1 Watt of input

power.  This normalization allows us to compare the directivity

of this antenna with that of an isotropic antenna driven by an

ideal source which has the same input power.  That is, for the

isotropic source there will be no source resistance.  To calculate

the scale factor, we first need to know the amount of power accepted

by the antenna.  This value was calculated in Section 3.4 as

1

POWERIN = - (Iin) (Iin*) [real(ZinAS) + Rs]

2

The scale factor is

SCALE = sqrt(WATTS / POWERIN)

where WATTS = 1 Watt of input power

sqrt() is the square root function.

Now we multiply all currents and voltages by the value of SCALE.

The gain of an isotropic antenna is given in dB as

Gain_Iso = 10.0 * log10(WATTS / (4 * PI * RADIUS**2))

where RADIUS is a distance sufficiently far from the antenna to

ensure the measurement is in the far-field.

Next, the pattern of the antenna is calculated as given in

the book (old eq. 6-52) using the appropriate direction cosines

for our geometry.  The front-to-back ratio is found by examination

of the pattern on boresight and 180 degrees from boresight.  The

front-to-sidelobe level is found by searching for the next

largest local maximum besides the main beam.  It is possible

that the main beam has split, and the front-to-sidelobe level is

negative (in dB).  The program then converts the currents and

voltages to dB at some phase angle in degrees (this is for

output purposes).  Finally, it finds the VSWR by computing

the reflection coefficient, ABS_GAM as

| ZinA - ZCin |

ABS_GAM = | ----------- |

| ZinA + ZCin |

where |z| represents the magnitude of the complex argument, z.

The VSWR is then

1 + ABS_GAM

VSWR = -------------

1 - ABS_GAM

This is the VSWR in the source transmission line relative to the

characteristic impedance of that line.

4.  SUBTLETIES AND ASSUMPTIONS

This section contains a list of assumptions and subtle

points associated with this design and analysis program.  For

further comments, please see Section 6., VERIFICATION AND

VALIDATION SUMMARY.

The technique used to find the antenna impedance matrix is

actually an approximation which [6] calls "significant."

Excitation of one element has an effect on another element

which in turn has an effect on a third, and so forth.  The method

used disregards the secondary effects on the third and

subsequent interactions.

By applying a unit voltage at element 1, we have normalized

the source voltage, Vin, located away from element 1 down the

source transmission line, to a value for which we later solve.

That is, for the purpose of calculation, we merely assume an

input current of 1 Amp and take care of the scaling later.

Notice that this routine can analyze other antenna arrays

as well.  For instance, proper modification of the transmission

line matrix, YT, and the antenna elements matrix, ZA, allow

us to analyze an array of dipoles located arbitrarily in space

whether or not they are connected.  Additional modifications to

the routine which finds the gain allows analysis of elements

other than dipoles.

In fact, [6] tells how to extend this program with very

little modification to include arrays of log-periodic dipole

arrays.

The excitation matrix, I = [1 0 0 ... 0]T can be used in

other useful ways.  For instance, changing the excitation matrix

to I = [ ... 0 0 0 1]T allows the user to investigate problems

arising from reflections at the termination.  I = [1 1 ... 1]T

allows the user to investigate arrays of independently driven

dipoles.  Of course, the transmission line matrix, YT, must be

removed (or set to 0) if the elements are not connected.

The input impedances can be confusing.  One could see

50 Ohms of input impedance as measured at the source (ZinAS)

while the input impedance seen at the shortest element of the

antenna is something completely different.  Therefore, one must

ensure that ALL impedance match for the highest efficiency.

That is, ideally Rs = 0, ZinAS = ZinA = Rin.

The optimal value of the termination impedance will change

with frequency.  If the antenna is radiating from the shortest

elements, Zout should equal ZO.  If the antenna is radiating from

the longest elements, then Zout approximately should equal Rin.

Since these values are close together, picking Zout = Rin should

be sufficient.

The gain of this antenna can be increased beyond that

predicted by Figure 11-13 and [11].  The classic LPDA antenna uses

a metal boom as a transmission line.  If the antenna transmission

line were allowed to meander slightly between elements, the phase of

the current could be forced to change precisely (not approximately)

by 180 degrees between elements.

5.  OUTPUT PARAMETERS

5.1.  DESCRIPTION

Many of the output parameters have already been described.

All input parameters (and quantities easily derivable, such as

Alpha from Sigma and Tau) are also output, so they will not be

discussed here.  The output depends on what analysis options

were chosen.  What follows is a list of all available outputs.

Voltages and currents at all elements, the source, and the

termination.

Gain pattern (in dBi) of the E-plane, H-plane, and a custom

plane for a single frequency.

Swept frequency analysis of the gain, front-to-back ratio,

front-to-sidelobe ratio, input impedance, and VSWR.

Single frequency analysis of the gain, front-to-back ratio,

front-to-sidelobe ratio, input impedance, and VSWR.

Characteristic impedance of the antenna transmission line.

5.2.  OUTPUT FILES

Log-Perd.FOR outputs several files depending on the options

chosen during the input portion of the program.  These files are

Log-Perd.INI An input file which is automatically updated

every time the program is executed.  If the file

does not exist in the current directory, it is

created using default values.

LP_DES.OUT   File containing a design summary.

LP_CPL.OUT   File containing a design summary and parameters

analyzed at a particular frequency and measured

in the custom plane.

LP_EPL.OUT   File containing a design summary and parameters

analyzed at a particular frequency and measured

in the E-plane.

LP_HPL.OUT   File containing a design summary and parameters

analyzed at a particular frequency and measured

in the H-plane.

LP_SWEPT.DAT File containing the result of a swept frequency

analysis and measured in the E-plane.

LP_GAIN.DAT  File containing two columns: frequency and gain

on boresight (dBi).  Suitable for use with the

plotting program.

LP_FTBR.DAT  File containing two columns: frequency and front

to back ratio (dB).  Suitable for use with the

plotting program.

LP_SRC.DAT   File containing two columns: frequency and

magnitude of the input impedance measured at the

source looking toward the antenna.

Suitable for use with the plotting program.

LP_Z_ANT.DAT File containing two columns: frequency and

magnitude of the input impedance measured at the

input terminals of the antenna (not at the source).

Suitable for use with the plotting program.

LP_VSWR.DAT  File containing two columns: frequency and VSWR

measured in the input line.  Suitable for use with

the plotting program.

LP_EPAT.DAT  File containing the E-plane gain (dBi) as a function

of angle.  Suitable for use with the plotting program.

LP_HPAT.DAT  File containing the H-plane gain (dBi)as a function

of angle.  Suitable for use with the plotting program.

LP_CPAT.DAT  File containing the C-plane gain (dBi) as a function

of angle.  Suitable for use with the plotting program.

6.  VERIFICATION AND VALIDATION SUMMARY

This program has undergone extensive, though not

exhaustive, testing to ensure that each module and the total

program work as they were intended.  However, no rigorous

testing has been performed which ensures that the models are

accurate (validation).  Instead, the outputs from this program

were casually compared to previously published results, such as

earlier editions of the book [9] and other sources already cited.

Furthermore, some hand waving, such as occurs in the description

of the program in the book on page 565, allows us to feel good about

the accuracy of the program without rigorously proving it.

Assumptions include

All conductors are lossless.

The medium surrounding the antenna has unity relative

permittivity and permeability.

All parts are perfectly manufactured and connected. In

particular, the feed is perfectly balanced.

The current distribution on each element is sinusoidal.

The antenna transmission line can be spaced precisely and

uniformly along its length.

The computer has no round-off error, especially with regard

to matrix inversion.

At the subroutine level, each module was checked with

simple test cases to verify its accuracy.

FINDST  Tested with sample inputs and compared to Figure

11.13 in the book.  Note that the book makes a 1 dB

correction to the results originally published[7].

It is known that there is a mistake in this

reference, and the amount of error varies from

about 0.5 to 2 dB.  Therefore, the optimum design as

calculated by this program will be slightly in

error.  This program could be used to find a

corrected version of Figure 11.13, to the accuracy

of the other error sources and assumptions.  This

exercise has been done, although with a different

program, and the results are published[11].

FINDZ   Tested by comparison of the results to identical

cases calculated in MathCAD which used the integral

equations of 7-29 and 7-39a.  As described in Part 4,

SUBTLETIES AND ASSUMPTIONS, an approximation is made

with regard to removing all elements except two:  the

excited one and the one under observation.

INPUT   Tested by direct observation of the results.

LUSOLV  Tested by sample test matrices[12].

LUDEC   Tested by sample test matrices[12].

OUTPUT1 Tested by direct observation of the results.

OUTPUT2 Tested by direct observation of the results.

PATTERN Tested indirectly by comparison of the analysis

results to published results[].

R2POL   Tested with sample inputs with the ouput compared to

hand calculations.

SICI    Tested with sample inputs and compared to [13]

SLL     Tested by direct observation of the results.

At the program level, the results were checked against

previously published results, against previously written

code in MathCAD, and against sanity checks.

7.  FORTRAN Compilation

The code contained in Log-Perd.FOR is written for the FORTRAN

77 language standard.  It was developed on a PC using Microsoft

FORTRAN Power Station for Windows and Windows 95 and has also been

successfully compiled on a Sun workstation.

8.  CREDITS

This program and all its subroutines were created by

Mr. Chris Bishop with the exception of the matrix inversion

routines, LUSOLV and LUDEC, and the sine and cosine integrals,

SI and CI.  The matrix inversion routines were created by

Dr. James T. Aberle, and the sine and cosine integrals were

created by Mr. Anastasis Polycarpou.  In both cases the routines

were made for the Telecommunications Research Center at Arizona

State University.

Biography

MR. CHRIS BISHOP received his bachelor's and master's degrees in

electrical engineering from the Georgia Institute of Technology

in 1990 and 1991, respectively.  His research focused precisely

on modeling log-periodic dipole arrays.  From 1992 through 1995

he worked for Phase IV Systems, Inc. testing radar seekers in

Hardware-in-the-Loop environments for US Army Missile Command.

His chief tasks there included analysis of existing electronics,

microwave devices, and radiating elements, as well as

specification of a compact antenna range.  Currently, Mr. Bishop

attends Arizona State University where he pursues the Ph.D. in

electrical engineering.

Acknowledgements

Mr. Bishop would like to thank Dr. Ed B. Joy of the Georgia

Institute of Technology for inspiring him to pursue antennas as

an area of research, Dr. Michael D. Fahey for making him realize

engineering is less of a job than it is a way of life, Dr. James

T. Aberle for allowing him time from his regular duties to write

the code for this application, and Dr. Constantine A. Balanis

for agreeing to include this code in his book.

9.  REFERENCES

[1]  V.H. Rumsey, "Frequency Independent Antennas."  1957 IRE

National Convention Record, pt 1, pp 114 - 118.

[2]  D.K. Cheng, FIELD AND WAVE ELECTROMAGNETICS, 2ed, Addison-

Wesley Publishing Co., Reading, Massachusetts, pp. 449-455,

1989.

[3]  S. Ramo, J.R. Whinnery, and T. van Duzer,  FIELDS AND WAVES

IN COMMUNICATION ELECTRONICS, 2ed, John Wiley & Sons,

New York, p 252, 1984.

[4]  G. De Vito and G.B. Stracca, "Further Comments on the Design

of Log-Periodic Dipole Antennas," IEEE Trans. Antennas

Propag., vol. AP-22, No. 5, pp. 714-718, September 1974.

[5]  R.C. Jasik and H. Jasik, HANDBOOK OF ANTENNA ENGINEERING,

McGraw-Hill, New York, 1984.

[6]  R.H. Kyle, "Mutual Coupling Between Log-Periodic Dipole

Antennas," General Electric Tech. Info. Series, Report No.

R69ELS-3, Chapter 2, December 1968.

[7]  R.L. Carrel, "Analysis and Design of the Log-Periodic Dipole

Antenna," Ph.D. Dissertation, Elec. Eng. Dept., University of

Illinois, 1961, University Microfilms, Inc., Ann Arbor,

Michigan.

[8]  M.T. Ma, THEORY AND APPLICATION OF ANTENNA ARRAYS, John Wiley

& Sons, New York, Chapter 5, 1974.

[9]  C.A. Balanis, ANTENNA THEORY ANALYSIS AND DESIGN, John Wiley

& Sons, New York, 1982.

[10] H.E. King, "Mutual Impedance of Unequal Length Antennas in

Echelon," IRE Trans. on Antennas and Propag., AP-5, pp 306-313,

July 1957.

[11] Y.T. Lo and S.W. Lee,  ANTENNA HANDBOOK THEORY, APPLICATIONS,

AND DESIGN, Van Nostrand Reinhold Company, New York, pp 9-23,

1988.

[12] J.R. Westlake, A HANDBOOK OF NUMERICAL MATRIX INVERSION AND

SOLUTION OF LINEAR EQUATIONS, John Wiley & Sons, New York,

Appendix C, 1968.

[13] M. Abramowitz and I. Stegun, editors, HANDBOOK OF MATHEMATICAL

FUNCTIONS, ninth printing, Dover Publications, New York, 1970.