c************************************************************

c    log_perd

c *************************************************************

c    This is a MATLAB and FORTRAN program, and it designs and analyzes

c     a log-periodic dipole array based on the geometry of Figure 11.9(a), the

c     data in Figures 11.13 and 11.14, and the design Equations of (11-28) - (11-34).

c

c    The user can tailor the design and analysis by following the listed steps

c    and selecting the appropriate options.

c  ……………………………………………………………………………...

c    For more details about the program, see the Log_Perd Document.doc

c   file included in the CD along with the log_perd computer program.

c  ……………………………………………………………………………..

c	**Design Procedure:

c

c	Step I:  Choose one of the following options:

c	.	Specify andThis fixes the directivity.

c	.	Specify the directivity.  This fixes and for optimum

c                   directivity design.

c	Step II:  Choose one of the following options:

c		.	Choose the boom spacing and boom diameter.  This sets the

c 	       characteristic impedance of the transmission line.

c		.	Choose the desired input impedance and boom diameter.  This

c	       sets the boom spacing.

c	Step III:  Choose one of the following options:

c		.	Round the calculated element diameters to the nearest

c	       available wire diameters.

c		.	Do not round the calculated element diameters.

c

c    After choosing the design options, proceed to the analysis.  The output

c    depends on analysis options chosen.

c

c	**Analysis Procedure:

c

c	Step IV: Select any combination of the following options:

c				 (do one, two, three or all of the following)

c		.	No analysis:

c		     OUTPUT: *Design summary (lists physical parameters and

c	                          geometry)

c		.	Analyze design at a single frequency:

c			OUTPUT: *Design summary (lists physical parameters and

c	                           geometry)

c				  *E- and H-plane pattern analysis

c				  *Critical parameters at analysis frequency

c		.	Analyze design at a single frequency

c			OUTPUT: *Design summary (lists physical parameters and

c	                           geometry)

c				   *Custom-plane pattern analysis

c				   *Critical parameters at analysis frequency

c		.	Analyze design over a frequency band:

c			OUTPUT: *Design summary (lists physical parameters and

c				   geometry)

c				   *Critical parameters as a function of frequency,

c					   including gain at boresight.

c

c    To complete the design and analysis of any option, the following input

c    and output parameters will generally be either specified or calculated.

c    Not all the parameters are required for all design and design options.

c

c	**Input parameters for array design:

c	1.	Title	=	Name of design

c	2.	Do	=	Desired maximum directivity (dBi)

c	3.	Fhigh	=	Upper design frequency (MHz)

c	4.	Flow	=	Lower design frequency (MHz)

c	5.	Rs	=	Source resistance (ohms)

c	6.	Zcin	=	Characteristic impedance of input line (ohms)

c	7.	Rin	=	Desired input impedance (real); typically

c				equal to characteristic impedance Zcin

c	8.	LLin	=	Length of the input transmission line; from

c				the source to first/shortest element (m)

c	9.	Zout	=	Termination impedance (ohms); typically equal

c				to characteristic impedance Zcin

c	10.	LLout	=	Length of the termination transmission line;

c				from the last-longest element to the load (m)

c	11.	LD	=	Desired length-to-diameter ratio of elements

c	12.	Navail	=	number of available element diameters (dimensionless)

c	13.	Davail	=	Diameters of available wires or tubes (cm)

c	14.	SB	=	Center-to-center boom spacing (cm)

c	15.	DB	=	Diameter of boom tubes or wires (cm)

c	16.	AFSEH	=	Analysis frequency for E- and H-patterns (MHz)

c	17.	AFSC	=	Analysis frequency for custom plane pattern (MHz)

c	18.	AFhigh	=	Highest frequency for swept frequency data (MHz)

c	19.	AFlow	=	Lowest frequency for swept frequency data (MHz)

c	20.	Phi	=	Angle of plane [(E-, H-) and/or custom] (degrees)

c

c	**Program output of array design:

c	1.	N	=	Number of antenna elements (integer)

c	2.	L	=	Lengths of antenna elements (m)

c	3.	ZL	=	Station (position) of each array element (m)

c	4.	D	=	Diameter of each array element (cm)

c	5.	VSWR	=	VSWR in input transmission line

c	6.	ZinA	=	Actual input impedance of design (ohms)

c	7.	DoA	=	Actual directivity of design along (dBi)

c	8.	PDo	=	Peak directivity along any axis (dBi)

c	9.	FTBR	=	Front-to-back ratio of amplitude pattern (dB)

c	10.	FSLL	=	Front-to-maximum side lobe level (dB)

c	11.	Iin	=	Current in input transmission line (A)

c	12.	Iel	=	Currents in array elements (A)

c	13.	Iout	=	Current in termination transmission line (A)

c	14.	Vin	=	Voltage at the source (V)

c	15.	Vel	=	Voltages of array elements (V)

c	16.	Vout	=	Voltage at the load (V)

c	17.	Epat	=	E-plane pattern (dBi)

c	18.	Hpat	=	H-plane pattern (dBi)

c	19.	Cpat	=	Any desired custom plane pattern (dBi)

c************************************************************