****************************************************************

%    Aperture

%   ************************************************************

%   This is a MATLAB program that calculates the radiation characteristics of a:

%   a.  Rectangular (of dimensions a and b)

%   b.  Circular (of radius a)

%   aperture antenna when it is either:

%   a. Mounted on an infinite perfect electric conducting (PEC) ground plane

%   b. Not mounted on a PEC ground plane

%

%   Each aperture has either one of the following aperture field distributions:

%   	a. Uniform

%   	b. Dominant mode:

%      		1.  TE10 for the rectangular

%      		2.  TE11 for the circular

%   ------------------------------------------------------------------------

%   The radiation characteristics calculated are based on the formulations

%   of Chapter 12, Sections 12.3-12.6, and Tables 12.1 and 12.2.

%   -------------------------------------------------------------------------

%   For each aperture, the following radiation characteristics are specified:

%

%   Input Parameters:

%

%    Aperture type:

% 	a. Rectangular

% 	b. Circular

%

%    Aperture dimensions

% 	a. Rectangular

%    		1. a (in lambda)

%    		2. b (in lambda)

% 	b. Circular

%    		1. Radius a (in lambda)

%

%    Aperture field distribution:

% 	a. Rectangular

%    		1. Uniform

%    		2. Dominant Mode (TE10)

% 	b. Circular

%    		1. Uniform

%    		2. Dominant Mode (TE11)

%

%    Aperture mounting:

% 	a. Infinite Perfect Electric Conductor (PEC)

% 	b. No Ground Plane

%

%   ----------------------------------------------------------------

%    Output Parameters:

%   ----------------------------------------------------------------

%    Aperture efficiency

%   ---------------------------------------------------------------

%    Approximate Directivity D0 (dimensionless and in dB) - Table 12.1

%    Approximate HPBW  for E-plane (in degrees) - Table 12.1

%    Approximate HPBW  for H-plane (in degrees) - Table 12.1

%    Approximate FNBW  for E-plane (in degrees) - Table 12.1

%    Approximate FNBW  for H-plane (in degrees) - Table 12.1

%   ----------------------------------------------------------------

%    Radiated power (watts) - Integration

%    Numerical Directivity D0 (dimensionless and in dB) - Integration

%    Numerical HPBW  for E-plane (in degrees) - Integration

%    Numerical HPBW  for H-plane (in degrees) - Integration

%    Numerical FNBW  for E-plane (in degrees) - Integration

%    Numerical FNBW  for H-plane (in degrees) - Integration

%   ----------------------------------------------------------------

%    Plots for both rectangular and circular apertures:

%	a. E-Plane normalized far-field amplitude pattern (in dB)

% 	b. H-plane normalized far-field amplitude pattern (in dB)

% 	c. E-plane directivity pattern (in dB)

% 	d. H-plane directivity pattern (in dB)

%   ----------------------------------------------------------------

%    The program uses 'polar_dB" to plot the polar patterns.

%    ---------------------------------------------------------------

%   To get the correct output, follow the instructions below:

%   ----------------------------------------------------------------

%	Input:

%		Choice:

%			A: Rectangular [dimensions (a,b) (in lambda)]

%			B: Circular [a (in lambda)]

%		Choice: Distribution

%			A: Uniform

%			B: Dominant mode (Rectangular: TE10; Circular: TE11)

%		Choice:

%			A: Ground plane

%			B: No ground plane

%

%	Output:

%		Directivity (dimensionless and  in dB)

%			1. Approximate

%			2. Numerical (directivity program)

%		Aperture efficiency (%)

%		HPBW (in degrees)

%			1. E-plane

%				a. approximate

%				b. numerical (iterative)

%			2. H-plane

%				a. approximate

%				b. numerical (iterative)

%		FNBW (in degrees)

%			1. E-plane

%				a. approximate

%				b. numerical (iterative)

%			2. H-plane

%				a. approximate

%				b. numerical (iterative)

%		PLOT: (Polar: in dB)

%			1. Normalized amplitude pattern (0-60 dB)

%			2. Directivity pattern (60 dB dynamic range)

%

%**********************************************************************