**********************************************************

c   Microstrip

c ***********************************************************

c   This is a MATLAB based program that designs and computes

c   the antenna radiation characteristics of:

c

c  	I.	Rectangular

c  	II.	Circular

c

c   microstrip patch antennas based on the:

c 	a.  Cavity model

c      	b.  Dominant mode

c   operation of each.  That is:

c  	a.  TM010 mode for the rectangular patch

c  	b.  TM011 mode for the circular patch

c

c	**Input parameters:

c	1.	Freq	=	Resonant frequency fr  (in GHz)

c	2.	Epsr	=	Dielectric constant of the substrate  r

c	3.	Height	=	height of the substrate h (in cm)

c	4.	y0	=	position of recessed feed point yo (in cm)

c				relative to leading radiating edge of

c				rectangular patch; not necessary for circular patch.

c

c	**Output parameters:

c

c	A.	Rectangular patch:

c

c		1.	Physical width of patch W (in cm)

c		2.	Effective length of patch Le (in cm)

c		3.	Physical length of patch L (in cm)

c		4.	Normalized E-plane amplitude pattern (in dB)

c		5.	Normalized H-plane amplitude pattern (in dB)

c		6.	E-plane half-power beamwidth (in degrees)

c		7.	H-plane half-power beamwidth (in degrees)

c		8.	Directivity (dimensionless and in dB)

c		9.	Resonant input resistance (in ohms)

c	a.	At leading radiating edge (y = 0)

c	b.	At recessed feed point from leading radiating edge (y=yo)

c

c	B.	Circular patch:

c

c		1.	Physical radius of the patch a (in cm)

c		2.	Effective radius of the patch ae (in cm)

c		3.	Normalized E-plane amplitude pattern (in dB)

c		4.	Normalized H-plane amplitude pattern (in dB)

c		5.	E-plane half-power beamwidth (in degrees)

c		6.	H-plane half-power beamwidth (in degrees)

c		7.	Directivity (dimensionless and in dB)

c

************************************************************