- ELEC ENG – ECE756 Design of Lightwave Communication Systems and Networks

- INSTRUCTOR NAME

- S. Kumar

DURATION OF EXAMINATION: 3 hours                                                  April 16, 2009

McMASTER UNIVERSITY FINAL EXAMINATION

THIS EXAMINATION PAPER INCLUDES 5 PAGES AND 6 QUESTIONS. YOU ARE RESPONSIBLE FOR ENSURING THAT YOUR COPY OF THE PAPER IS COMPLETE. BRING ANY DISCREPANCY TO THE ATTENTION OF YOUR INVIGILATOR.

Special Instructions:

Use of any calculator is allowed.

Answer all the questions.

- Good luck and have a great summer!

- Electrical field intensity in a medium is given by the three dimensional wave equation

∂2φ∂x2+∂2φ∂y2+∂2φ∂z2−1v2∂2φ∂t2=0.

Show that its solution is

φ(x,t)=f(t+αx+βy+γz),

where f is an arbitrary function and

v2(α2+β2+γ2)=1                                                                                       4 marks

2a. The average optical power output of the transmitter shown in the figure is 2 mW.  The fiber-optic link consists of a transmission fiber (TF) followed by a dispersion compensation fiber (DCF). The transmission fiber has a dispersion coefficient (β2 ) = -21 ps2 /km, loss coefficient = 0.25 dB/km and length = 50 Km. The DCF has a loss coefficient of 0.3 dB/km. The output of the DCF is connected to an amplifier of gain G= 18 dB. (a) Find the length of DCF so that the average output power of the amplifier is 3 mW. (b) Find the dispersion coefficient (β2 ) of the DCF such that the pulse width (FWHM) at the output of DCF is twice the pulse width (FWHM) at the input of the TF. Assume that a Gaussian pulse with FWHM = 12.5 ps is incident on the TF. Ignore β1 .

7 marks

Tx               TF        DCF         Amp                  Rx  Rx

(Hint: Suppose a Gaussian pulse with half-width at 1/e-intensity point T0 is incident on a fiber system with the transfer function

H(ω)=exp(jXω22),

electric field envelope at the output is given by              s o(t)=AT0T1exp[−t2(2T12)],   T1=T02−jX   )

2b. Explain the meaning of (a) numerical aperture and (b) Rayleigh scattering in optical fibers.                                                                                                              4 marks

3a. A 250 micron long InGaAsP  laser has an internal loss =  40 cm-1 , mirror loss =

100 cm-1, refractive index = 3.4. Calculate (a) photon life time and (b) threshold

electron density. Assume that gain varies as G = G0(N-N0) with G0 = 2.64 x 10-12

m3/s    and N0 =  0.5 x 1024 m-3.                                                                   5  marks

3b. In a direct band-gap material, an electron in the conduction band having a crystal momentum of 7.84 10−26 Kgm/s makes a transition to the valence band emitting electromagnetic wave of wavelength 0.8 micron. Calculate the band-gap energy.  Assume that the effective mass of electron in the conduction band is 0.07m and that in the valence band is 0.5m where m is the electron rest mass. Assume parabolic conduction and valence bands.                                                                                                   5 marks

4a. A photodiode has the following parameters:

Time constant of RC circuit (τRC) = 0.5 ps

Drift speed vd = 105 m/s

Absorption coefficient α = 2 x 106 m-1.

This photodiode is used in a receiver circuit of an optical transmission system operating at 10 Gb/s. It is desired that the bandwidth should be > 10 GHz and quantum efficiency η > 0.8. Find the range for depletion layer width w that satisfies the above design constraints.                                                                                                         5 marks

4b. Show that the SNR of a PIN photo-detector in the presence of shot noise and thermal noise is given by

SNR=(RP0)22qRP0Δf+4kBTΔfRL

where R is the responsivity, P0 is the incident power, Δf=effective bandwidth  , RL=load resistance, T= absolute temperature.                                                5 marks

5a. Two amplifiers with gain G1 and G2 are connected in tandem as shown in the figure. Derive a mathematical expression for the variance of the noise current due to signal-amplifier noise beating.  Assume that the spontaneous emission factor nsp is unity for both amplifiers and the power spectral density of the amplifier j is

PSDj(f)=hf(Gj−1),         j=1,2.

where  h=Planck’s constant, f = optical frequency. Assume that Δfis one-sided

bandwidth of a receiver (assumed to be ideal low pass filter) and Pin is the transmitter output power.                                                                                                  5 marks

Tx                   Amp1          Amp2              Rx

G1

G2

- 5b. Explain the difference between direct detection and coherent detection schemes. Can you detect the PSK signal using a coherent detector?                                           4 marks

6. The input power of a MZ modulator is P. It is desired that the output power of a MZM is P for bit ‘1’ and P/2 for bit ‘0’. The message signal, m(t) to the MZ modulator is a polar NRZ with A volts for bit ‘1’ and –A volts for bit ‘0’.  If the half-wave voltage, Vπ= 3 V, find A and the bias voltage. Assume that the MZ modulator is lossless and chirp-free. Choose A and bias voltage such that their absolute values are the lowest (for minimum power).                                                                                               6 marks

- Useful information

Speed of light in vacuum          c                        2.99793108m/s

Electron charge                         q                        1.60218 10−19C

Electron rest mass                     m                        9.109  10−31Kg

Planck’s constant                      h                         6.62510−34J.s

Boltzmann’s constant               kB                        1.3805410−23J/K

1 eV = 1.6021810−19 J.

- The power in dBm unit is given by

PdBm=10log10[P(mW)1mW]

- Numerical Aperture, NA = n12−n22

n1=core index, and n2= cladding index.

- The transfer function of a single mode optical fiber is given by

H(ω)=exp(jβ1ωL+jβ2ω2L2)

- For Gaussian pulse

TFWHM≈1.665T0

T0=half width at 1/e-intensity point.

The quantum efficiency η and responsivity R of a photodiode are related by

R=ηqhf

- The power spectral density (PSD) of shot noise is

Ss(f)=qIpRL

- and that of thermal noise is

ST(f)=2kBT

- Total loss in the cavity is given by

αcav=αint+12Lln1R1R2

- Photon life time is

τp=1(vαcav)

v is velocity of light in the medium.

- The rate equations governing photon density and electron density are given by

dφdt=Gmφ−φτp

dNdt=−Gmφ−Nτc+Jqd

- The threshold current density is

Jth= qdNthτc

- Optical power generated in a semiconductor laser is

P=(I-Ith)hfτp v/(qL)

- Energy density = photon density * h*f

- Optical intensity = energy density * v

- MZ modulator transfer function:

- Aout=Aincosm(t)πVπ−Vbiasπ2Vπ

- THE END